## Village of New London Water Plant OH3902611 (3955252)

## Drinking Water Consumer Confidence Report

For 2020

Section 1: Introduction

The Village of New London has prepared the following report to provide information

to you, the consumer, on the quality of our drinking water.  Included within this report is general health information, water quality test results, how to participate in decisions concerning your drinking water and water system contacts. We have a current, unconditioned license to operate our water system.

Section 2: Source Water Information.

The Village of New London pumps water into the New London Reservoir from Buck Creek, a branch of the Vermilion River. The reservoir has a surface area of 220 acres, when full, and has a volume of 1.4 billion gallons.  The average daily production of safe drinking water for 2020 was 521,000 gallons.

The Ohio EPA compiled a Drinking Water Source Assessment Report for the Village of New London with the assistance of John Chapin, Supt. of water, intended to identify drinking water protection areas and provide information on how to reduce the risk of contamination of the water within those areas.  A copy of this report may be obtained by contacting the Village of New London (419)-929-4091.

For the purposes of source water assessments, all surface waters are considered to be susceptible to contamination. By their nature surface waters are accessible and can be readily contaminated by chemicals and pathogens, with relatively short travel times from source to the intake. Based on the information compiled for this assessment, the Village of New London drinking water source protection area is susceptible to agricultural runoff, confined animal feedlots, pasture, silage, pesticide/ fertilizer/ petroleum storage, above ground storage tanks, industrial storm water, gas line rupture, gas station runoff, home construction runoff, feed lot runoff, marina boat docks, furniture repair and refinishing, unsewered areas, and truck fleet terminals. It is important to note that this assessment is based on available data, and therefore may not reflect current conditions in all cases. Water quality, land uses and other activities that are potential sources of contamination may change with time. While the source water for the Village of New London Public Water System is considered susceptible to contamination, historically, the Village of New London Public Water System has effectively treated this source water to meet drinking water quality standards.

Section 3: What are the sources of contamination to drinking water?

The sources of drinking water both tap water and bottled water includes rivers, lakes, streams, ponds, reservoirs, springs, and wells. As water travels over the surface of the land or through the ground, it dissolves naturally occurring minerals and, in some cases, radioactive material, and can pick up substances resulting from the presence of animals or from human activity. Contaminants that may be present in source water include: (A) Microbial contaminants, such as viruses and bacteria, which may come from sewage treatment plants, septic systems, agricultural livestock operations and wildlife; (B) Inorganic contaminants, such as salts and metals, which can be naturally-occurring or result from urban storm water runoff, industrial or domestic wastewater discharges, oil and gas production, mining, or farming; (C) Pesticides and herbicides, which may come from a variety of sources such as agriculture, urban storm water runoff, and residential uses; (D) Organic chemical contaminants, including synthetic and volatile organic chemicals, which are by-products of industrial processes and petroleum production, and can also come from gas stations, urban storm water runoff, and septic systems; (E) radioactive contaminants, which can be naturally-occurring or be the result of oil and gas production and mining activities. PFAS: Per- and polyfluoroalkyl substances (PFAS) are a group of man-made chemicals applied to many industrial, commercial and consumer products to make them waterproof, stain resistant, or nonstick. PFAS are also used in products like cosmetics, fast food packaging, and a type of firefighting foam called aqueous film forming foam (AFFF) which are used mainly on large spills of flammable liquids, such as jet fuel. PFAS are classified as contaminants of emerging concern, meaning that research into the harm they may cause to human health is still ongoing.

In order to ensure that tap water is safe to drink, USEPA prescribes regulations which limit the amount of certain contaminants in water provided by public water systems. FDA regulations establish limits for contaminants in bottled water, which must provide the same protection for public health. Drinking water, including bottled water, may reasonably be expected to contain at least small amounts of some contaminants. The presence of contaminants does not necessarily indicate that water poses a health risk. More information about contaminants and potential health effects can be obtained by calling the Environmental Protection Agency’s Safe Drinking Water Hotline (1-800-426-4791).

Section 4: Who needs to take special precautions?

Some people may be more vulnerable to contaminants in drinking water than the general population.  Immuno-compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/AIDS or other immune system disorders, some elderly, and infants can be particularly at risk from infection. These people should seek advice about drinking water from their health care providers. EPA/CDC guidelines on appropriate means to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available from the Safe Drinking Water Hotline (1-800-426-4791).

Section 5: About your drinking water.

The EPA requires regular sampling to ensure drinking water safety. The Village of New London conducted sampling for turbidity, pH, chlorine, alkalinity, hardness, inorganics, nitrates, synthetic organics, volatile organics, total organic carbon, dissolved organic compounds, and e-coli. Samples were collected for a total of approximately 180 different contaminants, most of which were not detected in the New London water supply. The Ohio EPA requires us to monitor for some contaminants less than once per year because the concentrations of these contaminants do not change frequently. Some of our data, though accurate, is more than one year old.

In 2020, our PWS was sampled as part of the State of Ohio’s Drinking Water Per- and Polyfluoroalkyl Substances (PFAS) Sampling Initiative. Six PFAS compounds were sampled, and none were detected in our finished drinking water. For mor information about PFAS, please visit pfas.ohio.gov.

Section 6: Table of Detected Contaminants

Listed below is information on those contaminants that were found in the Village of New London drinking water.

TABLE OF DETECTED CONTAMINANTS

| Contaminants (Units) | MCLG | MCL | Level Found | Range of Detections | Violation | Sample Year | Typical Source of Contaminants |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Radioactive Contaminants |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Alpha Emitters Radium 226 & Radium 228 | 0 0 | 15pCi/l 5pCi/l | 5.4 pCi/l 1.04 pCi/l | 5.4-5.4 pCi/l 1.04-1.04 pCi/l | No No | 2018 2018 | Erosion of natural deposits |
| Microbiological Contaminants |  |  |  |  |  |  |  |
| Turbidity (NTU) | n/a | TT | 0.34 99.9% | 0.04-0.34 | Yes | 2020 | Soil Runoff |
| Residual Disinfectants |  |  |  |  |  |  |  |
| RLCWA responsible for total coliform sampling |  |  |  |  |  |  |  |
| Inorganic Contaminants |  |  |  |  |  |  |  |
| Barium (ppb) | 2ppm | 2ppm | 0.0309 ppm | n/a | No | 2020 | Discharge of drilling wastes: discharge from metal refineries: erosion of natural deposits |
| Fluoride (ppm) | 4.0 | 4.0ppm | 0.83 ppm | 0.86-1.29 | No | 2020 | Erosion of natural deposits; Water additive, which promotes strong teeth; Discharge from fertilizer and aluminum factories |
| Nitrate (ppm) | 10 | 10ppm | .69 | <0.50-.69 | No | 2020 | Runoff from fertilizer use; Leaching from septic tanks, sewage, erosion of natural deposits. |
| Volatile Organic Contaminants |  |  |  |  |  |  |  |
| RLCWA responsible for TTHM & HAA5 samples |  |  |  |  |  |  |  |
| TOC | n/a | TT | 1.00 | 1.00-1.40 | No | 2020 | Naturally present in the environment |

Section 7: Turbidity is a measure of the cloudiness of water and is an indication of the effectiveness of our filtration system. Turbidity has no health effects, however, turbidity can interfere with disinfection and provide medium for microbial growth. Turbidity may indicate the presence of disease-causing organisms.  These organisms include bacteria, viruses, and parasites that can cause symptoms such as nausea, cramps, diarrhea, and can be associated with headaches. The turbidity limit set by the EPA is 0.3 in 95% of the daily samples and shall not exceed 1.0 NTU at any time. As reported above the Village of New London’s highest recorded turbidity result for 2020 was 0.34 NTU and lowest monthly percentage of samples meeting the turbidity limits was 99.9%.

Total organic carbon (TOC) has no health effects, however, it does provide a medium for the formation of disinfection by-products. These by-products include trihalomethanes (THM) and haloacetic acids (HAA’s). Some people who drink water containing THM’s in excess of the MCL over many years may experience problems with their liver, kidneys, or central nervous systems, and may have an increased risk of getting cancer.

If present, elevated levels of lead can cause serious health problems, especially for pregnant women and young children. Lead in drinking water is primarily from materials and components associated with service lines and home plumbing. The Village of New London is responsible for providing high quality drinking water but cannot control the variety of materials used in plumbing components. When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using water for drinking or cooking. If you are concerned about lead in your water, you may wish to have your water tested. Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline at 800-426-4791or at http://www.epa.gov/safewater/lead.

Alpha Emitters: Certain minerals are radioactive and may emit a form of radiation known as alpha

radiation.  Some people who drink water containing alpha emitters, in excess of the MCL, over many years may have an increased risk of getting cancer.

Combined Radium 226/228: Some people who drink water containing radium 226 or 228 in excess

of the MCL over any years may have an increased risk of getting cancer.

Section 8: Definitions of some terms contained within this report.

Maximum Contaminant Level Goal (MCLG): The level of a contaminant in drinking water, below which there is no known or expected risk to health.  MCLGs allow for a margin of safety.

Maximum Contaminant level (MCL): The highest level of contaminant that is allowed in drinking water.  MCLs are set as close to the MCLGs as feasible using the best available treatment technology.

Parts per Million (ppm) or Milligrams per Liter (mg/L) are units of measure for concentration of a contaminant.  A part per million corresponds to one second in a little over 11.5 days.

Parts per Billion (ppb) or Micrograms per Liter (μg/L) are units of measure for concentration of a contaminant.  A part per billion corresponds to one second in 31.7 years.

Picocuries per liter (pCi/L):  A common measure of radioactivity.

Maximum Residual Disinfectant Level Goal (MRDLG): The level of drinking water disinfectant below which there is no known or expected risk to health.  MRDLGs do not reflect the benefits of the use of disinfectants to control microbial contaminants.

Maximum Residual Disinfectant Level (MRDL): The highest level of a disinfectant allowed in drinking water.  There is convincing evidence that addition of a disinfectant is necessary for control of microbial contaminants.

Action Level (AL): The concentration of a contaminant, which if exceeded, triggers treatment or other requirements, which a water system must follow.

Treatment Technique (TT):  A required process intended to reduce the level of a contaminant in drinking water.

The “<” symbol: A symbol which means less than.  A result of <5 means that the lowest level that could be detected was 5 and the contaminant in that sample was not detected.

TOC value reported:  The value reported under “Level Found” for Total Organic Carbon (TOC) is the lowest ration between percentage of TOC actually removed to the percentage of TOC required to be removed.  A value of greater than one (1) indicates that the water system is in compliance with TOC removal requirements.  A value of less than 1 indicates a violation of the TOC removal requirements.

### Section 9:  How do I participate in decisions concerning my drinking water?

Public participation and comment are encouraged at regular meetings of the New London Village Council, which meets on the second and fourth Monday of every month at 7:00p.m. in the New London Community Center and Council Chambers at the Village Office, 115 E. Main St., New London, Ohio.  For more information on your drinking water contact:  John R. Chapin, Water Superintendent (419) 929-8419.  You can view the 2020 CCR at www.newlondonohio.com/ccr.html or request a paper copy at (419) 929-4091.  This report does not contain information on the water quality from R.L.C.W.A. but a copy of their consumer confidence report can be obtained by contacting them at:  Rural Lorain County Water Authority, 42401 SR 303, P.O. Box 567, Lagrange, Ohio 44050 or calling 800-842-1339.

Section 10: Violation description and health effects information for MCL exceedances, treatment technique, contact time violations, & action level exceedances.

In order to ensure disinfection, water in the treatment plant must be in contact with chlorine for a minimum amount of time. In December of 2020 the Village of New London failed to do this on three separate occasions. Inadequately treated water may contain disease-causing organisms. These organisms include bacteria, viruses, and parasites which can cause symptoms such as nausea, cramps, diarrhea, and associated headaches. People with severely compromised immune systems, infants, and some elderly people may be at increased risk. These people should seek advice about drinking water from their health care providers. The Village of New London has taken the following steps to correct this violation and prevent future violations from occurring: A chlorine analyzer has been placed on the treated influent water going into the clearwell. If unchlorinated water is entering the clearwell, the operator will be notified before the entire volume of water is affected. This will prevent us from not meeting our contact time in the future.